- Using Bluetooth APP

For iOS system:

- Open the APP store

- Click to search keyestudio, and you will see the keyes BT car.

- Tap to open the keyes BT car

- To open Bluetooth, click the “Connect” on the upper left corner, searching and connecting Bluetooth.

- Tap the Desktop_Car icon to enter the control interface of desktop car.

- Click the music icon to open the music control interface.

Below we have listed each button on the Bluetooth APP and what each button features.

| APP Buttons | Functions |  |
| --- | --- | --- |
|  | pair and connect HM-10 Bluetooth module |  |
|  | enter Bluetooth control interface |  |
|  | disconnect the Bluetooth |  |
|  | Control character | Function |
|  | Press: F Release: S | Press the button, robot goes front; release to stop forward |
|  | Press: B Release: S | Press the button, robot goes back; release to stop backward |
|  | Press: L Release: S | Press the button, robot turns left; release to stop |
|  | Press: R Release: S | Press the button, robot turns right; release to stop |
|  | Click to send：W | Click the button, robot always goes straight at the fastest speed |
|  | Click to send：Z | Click the button, robot always goes back at the fastest speed |
|  | Press: Q Release: S | Pressed, robot rotates to the left |
|  | Press: E Release: S | Pressed, robot rotates to the right |
|  | Click to send：S | Stop all the functions |
|  | - | Click to start the mobile direction sensing control; click again to exit the function |
|  | Click to send：Y | Start the obstacle avoiding function; click Stop to exit the function |
|  | - | Hold the button,and speak to cellphone:Go, desktop car will move forward; release and hold button again to speak: Stop, car will stop; back, go backward; left, turn left; right, turn right. |
|  | Click to send：X | Start the line tracking function; click Stop to exit |
|  | Click to send：U | Start the ultrasonic follow function; click Stop to exit |
|  | - | Click to enter the music control interface |
| Click the music iconto enter the music control interface; Music control buttons are as below: |  |  |
| Buttons | Control character | Function |
|  | Press: 1 Release: S | Press to play the tone DO; release to stop playing |
|  | Press: 2 Release: S | Press to play the tone RE; release to stop playing |
|  | Press: 3 Release: S | Press to play the tone MI; release to stop playing |
|  | Press: 4 Release: S | Press to play the tone FA; release to stop playing |
|  | Press: 5 Release: S | Press to play the tone SO; release to stop playing |
|  | Press: 6 Release: S | Press to play the tone LA; release to stop playing |
|  | Press: 7 Release: S | Press to play the tone SI; release to stop playing |
|  | Press: 8 Release: S | Press to play the tone DO; release to stop playing |
|  | Press: P Release: S | Press to play a song Happy Birthday; release to stop playing |
|  | return to the previous step |  |